Homework 6

Image Restoration

2020311381 Sanghyeok Choi

Goal of Project : Understanding on Optimal Filters for Image Restoration.

Tools : Inverse Filter, Wiener Filter, and Constrained Matrix Inversion Filter

1. Inverse Filter :

Let’s consider ‘f’ is original image. Image ‘f’ become degradation image ‘g’ by some point spread function ‘h’. Then you can think g = h*f. In frequency domain, it can be shown as G = HF. Now you know point spread function and degradation image ‘g’. You can get original image’s frequency domain by GH. To avoid the amplification of noise, apply not use as filter factor 1Hu,v, but a windowed version of it, cutting it off at a frequency before H(u,v) becomes too small or before its first zero. So F =GHu2+v2≤ m0Gu2+v2> m0

2. Wiener Filter :

In inverse filter, it never consider error. In this case, let’s consider error. Then g = h*f + γ.

Then G = HF + Γ. You don’t know exact error, so you cannot remove Γ. But it is white gaussian noise, so it can be apply ‘mean’. Let’s assume that identify filter m for f = m*g.

Rfgr,s = E( f(r)g(s) ) = E( -∞∞mr-r'gr'dr'gs ) = -∞∞mr-r'Egr'gsdr' ) =

-∞∞mr-r'Rggr',sdr' =m(r)*Rggr,s. Let’s convert to frequency domain then

Sfg = MSgg, M = SfgSgg = H*SffH2Sff+Sγγ. When you get Sgg, consider Rfγ = E( f(r)γ(s) ) = E(f(r))×E(γ(s)) = 0.

Now M = H*SffH2Sff+Sγγ = H*H2+SγγSff = H*H2+Γ. Assume Γ for constant because it is not big.

3. Constrained Matrix Inversion Filter :

In this case, we need find that h when ||g - h*f||2 is minimum. But consider there is error, add normalize factor in error;

|| l*f ||2. l is Laplacian matrix, for convert f to second derivative for smoothing constraint term.

Also second derivative can reduce difference between data. So find f when

ϵ = ||g - h*f||2 + γ|| l*f ||2 is minimum. You can find f when error is minimum by dϵdf = 0.

dϵdf =  dgTg-fThTg-gThf+fThThf+γfTlTlfdf.

Before calculation, d(fTAf)df = dfT(Af)df + d(ATf)T fdf = Af + ATf = (A +AT)f.

dϵdf =  dgTg-fThTg-gThf+fThThf+γfTlTlfdf = -2hTg+2hThf+2γlTlf = 0.

Finally f=HTH+γlTl-1HTg. Now let’s see l.

When you apply Fourier transform in l. How to apply Fourier transform in discussion part.

Lu,v=-5+Nδu+e-2πjvN+e-2πjN-1vN.

Lu,v’s magnitude =

2 + N-52+2N-5cos2πnN+2N-5cos2πN-1nN+2cos2πN-2nN   m=027 - 10cos2πnN - 10cos2πN-1nN+2cos2πN-2nN                                                      m≠0

So F = H*GH2+γL2. Apply inverse Fourier transform for get original image.

Process :

< Fig 1 >

There are five pictures in < Fig 1 >; Original image, Cylinder blurred image, Cylinder gaussian noised image, Real blurred image and Real gaussian noised image. They are made from moving 10 pixels in  direction.

g(x,y) = 1iTk=0iT-1f(i-k,j) = 110k=09f(i-k,j). Then G(u,v) = i=-∞∞j=-∞∞110k=09fi-k,je-2πjuxN+vyN = 110k=09i=-∞∞j=-∞∞fi-k,je-2πjuxN+vyN = 110k=09Fu,ve-2πjuxN = F(u,v) 110k=09e-2πjuxN = F(u,v)110sin10uπNsinuπNe-9πujN. So impulse response in frequency domain is H(u,v) = 110sin10uπNsinuπNe-9πujN.

Image size is 498 x 512 in first row of < Fig 1 > and 498 x 498 in second row of < Fig 2 >

I apply this method by “Matlab” tool.

Result :

Prob 1. Inverse Filter

< Fig 2 >

It is obvious from this that if we wish to obtain a real image, whatever we do to frequencies lower than m0 that lowest value of H could be 0, we must also do it to the frequencies higher than N −m0. H(u,v) = 110sin10uπNsinuπNe-9πujN, that mean m0 ≈ N10 ≈ 50~51. When you see second & fourth row, they seem restore well but still have some specific artifacts. Also in first row and third row, images are deformed.

Prob 2. Wiener Filter

< Fig 3 >

Wiener filter is optimized for noise added image. It combine deblurring filter and denosing filter. When you see first row and third row of < Fig 3 >, SNR seems better than previous one.

Prob 3. Constrained Matrix Inversion Filter

< Fig 4 >

It seems like Wiener filter. It contains specific terms that matrix linear equation’s constraint term to calculate. That smoothing constraint term acts like low-pass filter. As you can see < Fig 4 > looks like < Fig 3 >.

Discussion :

< Fig 5 >

Actually I think there are some error in textbook. Laplace filter is   0101-41010. When it convert to N x N, it is -4111⋅⋅⋅⋅1100000000000⋅⋅⋅⋅⋅⋅000000100000. So when you apply Fourier transform and its magnitude;

2 + N-52+2N-5cos2πnN+2N-5cos2πN-1nN+2cos2πN-2nN   m=027 - 10cos2πnN - 10cos2πN-1nN+2cos2πN-2nN                                                      m≠0

However, it has to correct by -4100⋅⋅⋅⋅1100000000000⋅⋅⋅⋅⋅⋅000000100000. So when you apply Fourier transform; -4 +e-2πjuN + e-2πjvN + e-2πj(N-1)uN + e-2πj(N-1)vN.

< Fig 5 > is apply real “Constrained Matrix Inversion Filter” that I think. When you apply filter, every images become blur. But I think noises are much more removed.

I think textbook is wrong but author may think other things. So I distinguish to 2 ways.

Reference :

Chapter5 of TextbookFile